2013 Ford Taurus Longitudinal Control

MAKE SURE PANDA IS PLUGED IN AND POWERED ON BEFORE SWITCHING THE CAR ON!

Acceleration (via accelerator pedal override):

- Accelerator pedal override with CAN control command

  - Make sure the switch on the pedal override box is switched to “ON” and the E-Stop button is released (twist counter-clockwise to release).

  - Connect CAN communication device to the DB-9 connector  of the pedal override box (labeled “ACCEL OVERRIDE”),

  - No termination resistor necessary.

  - Must initially send control message with a raw value = 0 to enable override control.

  - Use CAN control message to command accelerator pedal position within the range of 0 to 100%

  - CAN control Message:

    - CAN Type: Std 11 bit

    - Arb ID: 098

    - Length: 1 byte

    - Acceleration control signal:

      - Byte 1

      - Scaling: {Raw Value}*0.424444 – 1

      - Units: %

Figure 1: Pedal override box.

Acceleration/Deceleration (via Panda MiM):

- Acceleration and Braking override via man-in-the-middle (MiM) control with Panda

  - Panda device must be plugged in and powered on before switching the car on

    - Powered on by releasing E-Stop button on Pedal override box

  - Panda MiM box must be connected to the pedal override E-Stop 2-pin connector to enable power to the Panda

  - Connect CAN communication device to the DB-9 connector  of the Panda man-in-the-middle box (labeled “CRUISE CNT OVERRIDE”)

  - No termination resistor necessary

  - Use CAN control message to command acceleration and braking rate within the range of -5 to 5 m/s2

  - CAN control message:

    - CAN Type: Std 11 bit

    - Arb ID: 099

    - Length: 2 byte

    - Acceleration Control Signal:

      - Signed 2’s complement integer

      - Scaling: {Raw Value}*0.001

      - Units: m/s2

Figure 2: Panda man-in-the-middle box.

Troubleshooting:

| Problem | Solution |
| --- | --- |
| Stock accelerator pedal does not work | - Check that the E-Stop button on the pedal override box is released and re-start the car - Check pedal override box connection to accelerator pedal in driver foot compartment - Remove pedal override box connections from accelerator pedal and connect pedal by itself - Clear fault codes from the car - Restart car after attempting any of the solutions and re-try |
| Accelerator pedal override does not work | - Check that the switch on the pedal override box is switched to the “ON” position - Must send a CAN command signal with raw value equal to zero to initialize pedal override control every time the car or the pedal override box is first turned on. - Check that control message has length of 1 and Arb ID: 098 |
| Acceleration/Brake override with Panda MiM does not work | - Check that the Panda is turned on - 2-Pin Deutsch connector must be plugged in to pedal override box “E-Stop” connector and E-Stop should be released - CAN control message must have Arb ID: 099 - CAN control message must be 2 bytes long and control signal should be within byte 1 and 2 of the message |

E-Stop Info:

- Must be released before the car is turned on

  - Turns on stock accelerator pedal and pedal override box

  - Turns on power to Panda man-in-the-middle override box

- E-Stop button disables:

  - Stock accelerator pedal operation

  - Accelerator pedal override control

  - Panda man-in-the-middle override of acceleration and braking

- Use only for emergency in case of longitudinal controller failure / unexpected behavior

- Can be used to switch off other equipment through the extra “E-Stop” connector

  - Creates open circuit between the two pins of the “E-Stop” connector when pressed